Customer Requirement Specification

Smart Wi-Fi Lock Specification

Ver 1.0

30 November, 2017

Document History

| Date | Version | Author | Changes |
| --- | --- | --- | --- |
| 30/11/2017 | 1.0 | SUN Wentao | Draft version for internal approve |

Table of Contents

- INTRODUCTION

  - Purpose

This document describes the “Smart Wi-Fi Lock” device intended to be developed for XXXXX.

This document contains

- General product description

- Customer requirements for this product.

  - Scope

The topics covered in these requirements represent the commercial expectations from the product.

These expectations include: product feature description, power supply, user interface, connectivity, mechanics design and product testing.

  - Definitions, Acronyms, Symbols and Abbreviations

| Abbreviation | Comment |
| --- | --- |

2. General Product Description

  - Feature

- Power Supply

- DC +12V power adapter

- Wi-Fi Protocols Support

- 802.11 b/g/n support.

- Wi-Fi Direct (P2P) support.

- P2P Discovery, P2P Group Owner mode, P2P Power Management.

- Infrastructure BSS Station mode / P2P mode / softAP mode support.

- Easy Wi-Fi Pairing

- Support Smart Config feature. User can pair the set with a router by a 3rd party APP with its source code from Wi-Fi module supplier.

- Can integrate the Smart Config feature with our own APP.

- No Power Mode & No Network Mode

- When the lock is out of power or cannot connect to server through Wi-Fi, user can open the door by traditional mechanical ways (by using the key).

  - Top level block diagram

Wi-Fi Module

MCU

Lock Drive Circuit

Door Contact Sensor

  - User Interface

i) Buttons

The set has one button for entering Wi-Fi pairing mode.

ii) LEDs

The set had two LEDs (Red & Blue).

- Red LED, indicating the power status.

- Blue LED, indicating the Wi-Fi status.

- Technical Specifications

  - Wi-Fi Specifications

3.2.1 Operation Distance

Operation distance >= 30 meters (Outdoor)

3.2.2 Security

- Irreversible random HASH encryption algorithm (Anti-sniffing).

- No plain text password will be transmitted over the internet.

  - Mechanics

3.4.1 PCB Shape & Size

  - Shape: Rectangle

  - Length: <= 100 mm

  - Width: <= 100 mm

  - Height: <= 50 mm

3.4.2 Position of Button & LEDs

- Refer to section 3.4.4.

3.4.3 Position of DC Power Jack

- Refer to section 3.4.4.

3.4.4 Product Appearance

Power Status

Button

DC +12V

Connect to the lock

Wi-Fi Status

Connect to contact switch

- Device Operations

This section describes control and behaviour for the set.

  - Button Operations & Finite States Machine

- SP – Short Press (< 2 sec)

- LP – Long Press (>= 2 sec & < 4 sec)

- VLP – Very Long Press (>= 4 sec)

Power OFF

Power On

(Auto connect to router)

Wi-Fi

Connected

Wi-Fi

Pairing

VLP

LP

SP

  - Visual Indicators

| LED Status |  |  |
| --- | --- | --- |
| Device Status | Wi-Fi LED Status (Blue) | Power LED Status (Red) |
| OFF | OFF | OFF |
| Connected | Solid ON | Solid ON |
| Pairing | 1Hz Blinking | Solid ON |

- Apps Control

- APP can send commands such as “lock” or “unlock” to the server.

- APP can get the door status from server.

- Device can receive the “lock” or “unlock” command from the server.

- Device can update the door status to the server.

- Hardware Testing

- The 3rd-party software named “TCP/IP Debugger” will be used for hardware sanity test and internal demo at product development stage.

- The “Smart Config” APP from Wi-Fi module supplier will be used for Wi-Fi pairing demo.

- Design Release

- Hardware: Schematic & PCB layout files. 2 sets of PCB boards with locks and adapters for demo.

- Firmware: All source code and SDKs/Compilers involved in the project.

- Documents:

    - Firmware release notes

    - All the original documents from module suppliers

    - Introduction of Irreversible random HASH encryption algorithm

- Open Points

| Section/Chapter | Description |
| --- | --- |

- Commercial

  - Planning

| Milestone | Items working on | Days (8 hours) |
| --- | --- | --- |
| Hardware Development | - Schematic design - PCB layout - PCB fabrication - PCBA - Hardware Sanity Test | 15 |
| Firmware Development | - Wi-Fi communication - Lock/unlock - Door contact switch - Password encryption algorithm implement & test | 10 |
| Co-Development | - Co-debug with APP and Server | 10 |

  - Quotation

| Items | Cost | Remark |
| --- | --- | --- |
| NRE | HK$20,000 | Including 2 sets of sample boards with locks and adapters |
| Design Files | HK$4,000 | Including schematics, PCB layout, firmware projects and source code, source code of 3rd party libraries if involved, and key algorithm documents, etc. |
| Total | HK$24,000 |  |
| Extra sample boards | N × HK$300 + N × HK$200 + HK$200 | Including PCBA, adapters and e-locks. Price of PCBA: HK$300 per set Price of adapter & e-lock: HK$200 per set MOQ: 5 sets Engineering fee: HK$200 e.g. Ordering 5 sets, cost = 5*300+5*200+200 = HK$2,700 e.g. Ordering 10 sets, cost = 10*300+5*200+200 = HK$4,200 |

  - Certifications

All necessary certification obtaining procedures including but not limited to RoHS, FCC, CE will be conducted by customer.